Research Summary: Paper 1

Title: Detecting ADS-B Spoofing Attacks using Deep Neural Networks

Date: June 18, 2026

Objective

The main goal of this paper is to stop hackers from messing with air traffic control systems. Because ADS-B signals are completely unencrypted, anyone with a cheap Software Defined Radio (SDR) can blast fake flight data into the air. This paper introduces a system called SODA to catch these fake signals by looking at the physical properties of the radio waves, rather than just reading the text data inside the message.

Methodology & Dataset

The researchers didn't just run simulations; they built a real testbed. They used a basic laptop, a cheap RTL-SDR dongle, and an antenna to collect 18,675 real, authentic flight messages in an open area.

Then, to create the "hacker data," they took a high-end SDR transmitter (a USRP B210) inside a shielded room (an anechoic chamber) to blast out three types of attacks without interfering with real planes:

- Message Replay: Recording a real plane and broadcasting it again later.

- IQ Data Replay: A stealthier replay that copies the exact radio frequency waveforms.

- Ghost Aircraft Injection: Fabricating completely fake flight paths out of thin air.

SODA Architecture

SODA stands for Spoofing Optimized Detector for ADS-B. It sits quietly between the antenna and the air traffic console, running incoming signals through a two-stage Deep Neural Network (DNN) pipeline:

- Stage 1: Message Classifier (The Filter)

  - What it looks at: Raw interleaved IQ samples (the raw digital radio signal).

  - Job: It acts as a gatekeeper to instantly check if the signal wave looks like it came from a real aircraft transponder or an amateur ground-based SDR spoofer. If it looks like an SDR, it gets flagged immediately.

- Stage 2: Aircraft Classifier (The Identity Checker)

  - What it looks at: The phase of the radio waves.It intentionally ignores signal strength/amplitude so a hacker can't trick it by turning the volume up or down.

  - Job: Every single aircraft transponder has tiny hardware imperfections from manufacturing that leave a unique "fingerprint" on the wave phase. This stage predicts exactly which specific plane sent the signal and cross-checks it against the text ID (ICAO address) written inside the packet. If the fingerprint doesn't match the ID, it catches the hacker.

Key Performance

- Stage 1 was incredibly accurate, catching ground-based spoofers 99.34% of the time with a tiny 0.43% false alarm rate. It completely beat older machine learning tricks like XGBoost and Support Vector Machines.

- Stage 2 successfully kept track of 238 individual planes, identifying them with 96.66% accuracy based purely on their radio signatures.

Limitations & Gaps (Our Agentic AI Opportunity)

Here is the catch: SODA does not generalize well to unknown attacks. The paper reveals that if you train the neural network only on basic message replays, its ability to catch a smart ghost aircraft injection drops down to just 21.80%. It completely relies on having seen that specific type of attack before during its training.

How We Can Use This in Our Lab

We can use their math for calculating phase patterns as a tool for our system. But instead of relying on a static neural network that panics when it sees a new type of hack, our Agentic AI will treat this anomaly score as just one piece of evidence. If the network gives a low confidence score, our AI Agent will autonomously spin up other tools—like checking if the plane's movements follow physical laws (Kalman filtering)—to catch smart, zero-day attacks that this paper's model would completely miss.